IP Fragmentation Example 1 Answer

An IP Packet of 8200 bytes (total length) is to be transmitted over a link with a frame size MTU of 512 bytes:

- How many bytes of data are to be transmitted? A: 8180 bytes 
- The total packet length given is 8200 which includes 20 bytes of IP header
- We then have 8200 - 20 = 8180 bytes of data to be transmitted.

- How many bytes of data can be carried in each fragment? A: 488 bytes
- The total length of the fragment, including 20 bytes of header, must be 512 bytes or less. 
- We can then carry a maximum of 512 - 20 = 492 bytes of data in a fragment.
- Now, because the offset value must be a multiple of 8, the amount of data bytes must be a multiple of 8
- 492 is NOT a multiple of 8, so we have to go with the closest lower multiple of 8, i.e 488 bytes

- How many IP fragments must be sent? A: 17
- We must transmit 8180 bytes of data, and can carry 488 (or less) bytes in a fragment
- 8180/488 = 16.76 
- We must send 16 fragments carrying 488 bytes each, plust a last one carrying the remaining
- Then, we must send 17 fragments in total.

- What is the offset value for the third IP fragment? A: 122
- The offset represents the relative position of the first byte in the fragment, expressed in multiples of 8
- The first fragment carries bytes 0 to 487. The offset in the first fragment is 0,
- The second fragment carries bytes 488 to 975. The offset in the second fragment is 488/8 = 61
- The third fragment carries bytes 976 to 146. The offset in the third fragment is 976/8 = 122

Another way to look at it is:
- The offset increment is the amount of data bytes carried divided by 8, i.e 488/8 = 61
- The offset in the first fragment is always 0
- The offset in the second fragment is 0+61 = 61
- The offset in the third fragment is 61+61 = 122

IP Fragmentation Example 2

An IP Packet carrying 40000 data bytes is to be transmitted over a link with a frame size MTU of 1480 bytes:

- How many bytes of data are to be transmitted? A: 40000

- How many bytes of data can be carried in each fragment? A: 1456 bytes
- We must consider that each fragment will have 20 bytes of header
- With an MTU of 1480, an IP packet can carry a max. of 1460 bytes of data
- Because the offset must be a multiple of 8, the amount of data bytes carried must also be a multiple of 8
- 1460 is NOT a multiple of 8, so we find the closest one
- Then, each fragment can carry 1456 bytes of data

- How many IP fragments must be sent? A: 28
- 40000/1456 = 27.48
- We must sent 27 fragments with 1456 bytes of data, plus 1 with the reminder. i.e. 28 fragments

- What is the offset value for the 4th IP fragment? A: 546
- The fragment data size is 1456, then the offset increment is 1456/8 = 182
- The Fragment 1 offset is 0, Fragment 2 offset is 182, Fragment 3 offset is 364
- Fragment 4 offset is 546

- A fragment is lost in the way. Will the IP layer retransmit the fragment? A: NO

IP is best effort, it does not keep track of packet to ensure they reach destination. Retransmissions are handled by the upper layers.

IP Fragmentation Example 3

An IP Packet with a total size of 12000 bytes is to be transmitted over a link with a frame size MTU of 1500  bytes:

- How many bytes of data are to be transmitted?

- How many bytes of data can be carried in each fragment?

- How many IP fragments must be sent?

- What is the offset increment?

### VLSM Scenario 1

You must optimally subnet the allocated major network, and answer the following questions:

- How many subnets are needed in total? 6

- Calculate the subnet address and the subnet mask for each network

Asgard: 212.125.8.0/26
New York: 212.125.8.64/27
New Jersey: 212.125.8.96/27
Vormir: 212.125.8.128/29
Link1: 212.125.8.136/30
Link2: 212.125.8.140/30

- What is the broadcast address for the Asgard network? 212.125.8.63

- What is the first usable IP for the the New Jersey network? 212.125.8.97

- What is the last usable IP address in the New York network? 212.125.8.94

### VLSM Scenario 2

You must optimally subnet the allocated major network, and answer the following questions:

- How many subnets are needed in total? 8 Subnets

- Calculate the subnet address and the subnet mask for each network

Nairobi: 136.15.0.0/19

Lisbon: 136.15.32.0/22

Stockholm: 136.15.36.0/23

Tokyo: 136.15.38.0/25

Berlin: 136.15.38.128/26

Rio: 136.15.38.192/26

Link 01: 136.15.39.0/30

Link 02: 136.15.39.4/30

3. What is the broadcast address for the Tokyo network? 136.15.38.127/25

4. What is the first usable IP for the the Rio network? 136.15.38.193/26

5. What is the last usable IP address in the Lisbon network? 136.15.35.254/22

6. What is the unused range? 136.15.39.8/30 to 136.15.39.255/30

### VLSM Scenario 3

- How many subnets are needed in total? 9

- Calculate the subnet address and the subnet mask for each network
North - 175.64.0.0/19
Reach - 175.64.32.0/20
Vale - 175.64.48.0/20
Riverlands - 175.64.64.0/21
Stormlands - 175.64.72.0/23
Westerlands - 175.64.74.0/25
Link1 - 175.64.74.128/30
Link2 - 175.64.74.132/30
Link3 - 175.64.74.136/30

- What is the broadcast address for the Riverlands network? 175.64.71.255

- What is the first usable IP for the the Westerlands network? 175.64.74.1

- What is the last usable IP address in the North network? 175.64.31.254

- What is the unused range? 175.64.74.140 - 175.64.127.255

### VLSM Scenario 4

- How many subnets are needed in total? 9

- Calculate the subnet address and the subnet mask for each network
ATC 136.186.224.0/21
EN 136.186.232.0/21
AD 136.186.240.0/22
LB 136.186.244.0/23
BA 136.186.246.0/23
UN 136.186.248.0/25
Link1 136.186.248.128/30
Link2 136.186.248.132/30
Link3 136.186.248.136/30

- What is the broadcast address for the ATC network? 136.186.231.255

- What is the first usable IP for the the LB network? 136.186.244.1

- What is the last usable IP address in the UN network? 136.186.248.126

- What is the unused range? 136.186.248.140 – 136.186.255.255

### VLSM Scenario 5

- How many subnets are needed in total?  9

- Calculate the subnet address and the subnet mask for each network

  - Exhibition - 45.90.64.0/21

  - Williams - 45.90.72.0/22

  - Bourke - 45.90.76.0/23

  - Flinders - 45.90.78.0/24

  - Lonsdale - 45.90.79.0/26

  - Collins - 45.90.79.64/28

  - Link 1 - 45.90.79.80/30

  - Link 2 - 45.90.79.84/30

  - Link 3 - 45.90.79.88/30

- What is the broadcast address for the Lonsdale network? 45.90.79.63

- What is the first usable IP for the the Collins network? 45.90.79.65

- What is the last usable IP address in the Williams network? 45.90.75.254

- What is the unused range? 45.90.79.92 - 45.90.127.255

### VLSM Scenario 6

- How many subnets are needed in total? There are 9 subnets

- Calculate the subnet address and the subnet mask for each network

  - Emporium: 103.5.128.0/19

  - Doncaster: 103.5.160.0/20

  - Chadstone: 103.5.176.0/22

  - Eastland: 103.5.180.0/23

  - Jam Factory: 103.5.182.0/27

  - Melbourne Central: 103.5.182.32/27

  - Link 1: 103.5.182.64/30

  - Link 2: 103.5.182.68/30

  - Link 3: 103.5.182.72/30

- What is the broadcast address for the Chadstone network? 103.5.179.255

- What is the first usable IP for the Doncaster network? 103.5.160.1

- What is the last usable IP address in the Emporium network? 103.5.159.254

- What is the unused range? 103.5.182.76 - 103.5.255.255

### VLSM Scenario 7

You must optimally subnet the allocated major network, and answer the following questions:

- How many subnets are needed in total? 8

- Calculate the subnet address and the subnet mask for each network
Bespin: 103.160.0.0/20
Corellia: 103.160.16.0/22
Ahch-To: 103.160.20.0/22
Hoth: 103.160.24.0/24
Jakku: 103.160.25.0/25
Tatooine: 103.160.25.128/26
Link 1: 103.160.25.192/30
Link 2: 103.160.25.196/30

- What is the broadcast address for the Tatooine network? 103.160.25.191

- What is the first usable IP for the the Jakku network? 103.160.25.1

- What is the last usable IP address in the Hoth network?103.160.24.254

- What is the unused range? 103.160.25.200 - 103.160.31.255

### VLSM Scenario 8

You must optimally subnet the allocated major network, and answer the following questions:

- How many subnets are needed in total?

- Calculate the subnet address and the subnet mask for each network

- What is the broadcast address for the Caracas network?

- What is the first usable IP for the the Maracay network?

- What is the last usable IP address in the Merida network?

- What is the unused range?